# Setup system

## I.1 Setup Manager Node.

Requirement: Install 5GC Core and LISP

### I.1.1 Install 5GC core.

Following document on https://free5gc.org/guide/3-install-free5gc/ :

- Install Golang Version 1.18.10

- # remove go

- sudo rm -rf /usr/local/go

- dpkg -l | grep golang

- sudo apt-get remove golang*

- # install go 1.18

- wget https://dl.google.com/go/go1.18.10.linux-amd64.tar.gz

- sudo tar -C /usr/local -zxvf go1.18.10.linux-amd64.tar.gz

- mkdir -p ~/go/{bin,pkg,src}

- # The following assume that your shell is bash:

- echo 'export GOPATH=$HOME/go' >> ~/.bashrc

- echo 'export GOROOT=/usr/local/go' >> ~/.bashrc

- echo 'export PATH=$PATH:$GOPATH/bin:$GOROOT/bin' >> ~/.bashrc

- echo 'export GO111MODULE=auto' >> ~/.bashrc

- source ~/.bashrc

- go version

- Control-plane Supporting Packages (mongodb version 4.4)

- # Install mongodb 4.4 for control-plane

- sudo apt -y update

- wget -qO - https://www.mongodb.org/static/pgp/server-4.4.asc | sudo apt-key add -

- echo "deb [ arch=amd64,arm64 ] https://repo.mongodb.org/apt/ubuntu $(lsb_release -cs)/mongodb-org/4.4 multiverse" | sudo tee /etc/apt/sources.list.d/mongodb-org-4.4.list

- sudo apt-get update

- sudo apt-get install -y mongodb-org

- sudo systemctl start mongod

- sudo systemctl enable mongod

- # User-plane Supporting Packages

- sudo apt -y update

- sudo apt -y install git gcc g++ cmake autoconf libtool pkg-config libmnl-dev libyaml-dev

- Clone the free5GC repository.

- # install control-plane

- cd ~

- git clone --recursive -b v3.3.0 -j `nproc` https://github.com/free5gc/free5gc.git

- Compile network function services in free5gc

- cd free5gc

- make

- # Install User Plane Function (UPF)

- git clone -b v0.8.2 https://github.com/free5gc/gtp5g.git

- cd gtp5g

- make

- sudo make install

- cd ~/free5gc

- make upf

- Install WebConsole

- # Note: 2GB or more of OS memory is recommended. WebConsole may be failed to build if memory is less then 1GB.

- sudo apt remove cmdtest yarn

- curl -sS https://dl.yarnpkg.com/debian/pubkey.gpg | sudo apt-key add -

- echo "deb https://dl.yarnpkg.com/debian/ stable main" | sudo tee /etc/apt/sources.list.d/yarn.list

- sudo apt update

- sudo apt install -y nodejs yarn

- cd ~/free5gc

- make webconsole

- Change 5GC config

- cd ~/free5gc

- At run.sh located in free5gc folder, command two lines for disable UPF.

- At config/amfcfg.yaml, change AMF IP address:

- At config/smfcfg.yaml, change SMF config like below:

- Change SMF code for getting UP information

At  /free5gc/NFs/smf/internal/context/datapath.go , add one more line (add code) and one more function Print()

At /free5gc/NFs/smf/internal/sbi/producer/oam.go , update PDUSessionInfor and httpResponse as follows:

### I.1.2 Install LISP control plane.

Follow the https://github.com/binhfdv/lispers-config to set up configurations

In Manager node: install lisp-ms

In Worker 1 and Worker 3: install lisp-etr for each one

Replace content in files

- src/lisp.py from the original codes with that of and lisp_v3.py

- src/lisp-etr.py from the original codes with that of lisp-etr-prometheus.py (remember to replace the Prometheus query in the codes with your own one in order to get the service resources)

- src/lisp-ms.py from the original codes with that of scr/lisp-ms.py

Notes: Ctrl + F to find term: “added code” to add api endpoints for dynamically changing lisp UI displaying values.

## I.2 Setup Worker2 Node.

Requirement: Install GNB & UE following document on https://github.com/aligungr/UERANSIM

- cd ~

- git clone https://github.com/aligungr/UERANSIM

- sudo apt update

- sudo apt upgrade

- sudo apt install make

- sudo apt install gcc

- sudo apt install g++

- sudo apt install libsctp-dev lksctp-tools

- sudo apt install iproute2

- sudo snap install cmake –classic

- cd ~/UERANSIM

- make

### I.2.1 Run GNB with GNB config file:

Change IP AMF

Chang ngapIP in GNB config file to IP which connects to AMF

Change gtpIP (10.100.200.3) in GNB config file to IP which connects to UPF

Then run GNB:

- ./build/nr-gnb -c config/free5gc-gnb.yaml

### I.2.2 Run UE with UE config file:

Change GNB IP in config/free5gc-ue.yaml

Then run UE:

- ./build/nr-ue -c config/free5gc-ue.yaml

## I.3 Setup Worker3/Worker1 Node.

Requirement: Install K8S cluster with service running as Anycast IP address.

### I.3.1 Install K8S cluster version 1.22.

- sudo apt-get install kubeadm=1.22.0-00 kubelet=1.22.0-00 kubectl=1.22.0-00 -y

- sudo kubeadm init --pod-network-cidr=10.244.0.0/16

- mkdir -p $HOME/.kube

- sudo cp -i /etc/kubernetes/admin.conf $HOME/.kube/config

- sudo chown $(id -u):$(id -g) $HOME/.kube/config

- kubectl apply -f https://raw.githubusercontent.com/coreos/flannel/master/Documentation/kube-flannel.yml

### I.3.2 Install service-a running in K8S cluster.

Deploy service deployment by service.yaml file

Deploy service by svc-deployment.yaml file. Cluster IP was setup with 10.96.10.10

## I.4 Setup Mininet Node.

Following instructions of Onos+P4 tutorial command: https://github.com/opennetworkinglab/ngsdn-tutorial

Clone P4-MUP-SRv6 repository and install required dependencies:

- cd P4-MUP-SRv6

- make deps

Start P4-MUP-SRv6 project:

- cd P4-MUP-SRv6

- make restart

Wait about 1 minute for warming up.

Inject network config file to Onos:

- make netcfg

Activate Onos application:

- make app-reload

Wait about 1 minute for warming up. Then go to Browser to check Onos system on http://192.168.20.11:8181/onos/ui/login.html#/topo2

- User: onos

- Password: rocks

Can monitor the network topology in real-time.

Install “bridge-utils” packet for Mininet container:

- docker exec -it  mininet sh

- echo "deb http://archive.debian.org/debian stretch main" > /etc/apt/sources.list && chmod 1777 /tmp && rm -rf /var/lib/apt/lists/* && apt-get update && apt install bridge-utils

Then Ctrl+D to exit.

- Connect worker2 to virtual Switch running inside Mininet container

Then create a bridge for linking virtual ethernet of host(v_eth1) to virtual ethernet of switch R1(v_eth3) running inside Mininet:

- docker exec -it  mininet brctl addbr mybridge

- ip link add v_eth1 type veth peer name v_eth2 && ifconfig v_eth1 up

- ip link set v_eth2 netns  $(docker inspect --format='{{ .State.Pid }}' $(docker ps -aqf "name=mininet"))

- docker exec -it  mininet ifconfig v_eth2 up

- docker exec -it  mininet brctl addif mybridge v_eth2

- docker exec -it  mininet brctl addif mybridge v_eth3

- docker exec -it  mininet ifconfig mybridge 10.100.200.19/24 up

- docker exec -it  mininet ping 10.100.200.20 -I mybridge -c 2

Then create a bridge for linking virtual ethernet of host(v_eth1) to physical ethernet of host(enxf8e43b4d17e5):

- ovs-vsctl del-br br0

- ovs-vsctl add-br br0

- ifconfig br0 up

- ovs-vsctl add-port br0 enxf8e43b4d17e5

- ovs-vsctl add-port br0 v_eth1

- ifconfig enxf8e43b4d17e5 0

- ifconfig br0 10.100.200.4/24 up

- ping 10.100.200.20 -I br0 -c 2

- we can connect worker2(10.100.200.3) to virtual host running on Mininet (10.100.200.20).

- Test by “ping 10.100.200.20” from worker2

- Connect worker3 to virtual Switch running inside Mininet container

Then create a bridge for linking virtual ethernet of host(v2_eth1) to virtual ethernet of switch R8(v2_eth3) running inside Mininet:

- ip link add v2_eth1 type veth peer name v2_eth2 && ifconfig v2_eth1 up

- ip link set v2_eth2 netns  $(docker inspect --format='{{ .State.Pid }}' $(docker ps -aqf "name=mininet"))

- docker exec -it  mininet brctl addbr mybridge2

- docker exec -it  mininet ifconfig v2_eth2 up

- docker exec -it  mininet brctl addif mybridge2 v2_eth2

- docker exec -it  mininet brctl addif mybridge2 v2_eth3

- docker exec -it  mininet ifconfig mybridge2 192.168.0.19/24 up

- docker exec -it  mininet ping 192.168.0.20 -I mybridge2 -c 2

Then create a bridge for linking virtual ethernet of host(v2_eth1) to physical ethernet of host(enxf8e43b26ac73):

- ovs-vsctl del-br br1

- ovs-vsctl add-br br1

- ifconfig br1 up

- ovs-vsctl add-port br1 enxf8e43b26ac73

- ovs-vsctl add-port br1 v2_eth1

- ifconfig enxf8e43b26ac73 0

- ifconfig br1 192.168.0.4/24 up

- ping 192.168.0.20 -I br1 -c 2

- we can connect worker3(192.168.0.3) to virtual host running on Mininet (192.168.0.20).

- Test by “ping 192.168.0.20” from worker3

- Connect worker1 to virtual Switch running inside Mininet container

Then create a bridge for linking virtual ethernet of host(v3_eth1) to virtual ethernet of switch R9(v3_eth3) running inside Mininet:

- ip link add v3_eth1 type veth peer name v3_eth2 && ifconfig v3_eth1 up

- ip link set v3_eth2 netns  $(docker inspect --format='{{ .State.Pid }}' $(docker ps -aqf "name=mininet"))

- docker exec -it  mininet brctl addbr mybridge3

- docker exec -it  mininet ifconfig v3_eth2 up

- docker exec -it  mininet brctl addif mybridge3 v3_eth2

- docker exec -it  mininet brctl addif mybridge3 v3_eth3

- docker exec -it  mininet ifconfig mybridge3 192.168.1.19/24 up

- docker exec -it  mininet ping 192.168.1.20 -I mybridge3 -c 2

Then create a bridge for linking virtual ethernet of host(v3_eth1) to physical ethernet of host(enxaaaaaaaaab7f):

- ovs-vsctl del-br br2

- ovs-vsctl add-br br2

- ifconfig br2 up

- ovs-vsctl add-port br2 enxaaaaaaaaab7f

- ovs-vsctl add-port br2 v3_eth1

- ifconfig enxaaaaaaaaab7f 0

- ifconfig br2 192.168.1.4/24 up

- ping 192.168.1.20 -I br2 -c 2

- we can connect worker1(192.168.1.3) to virtual host running on Mininet (192.168.1.20).

- Test by “ping 192.168.1.20” from worker1

# Running system

- Step 1: RUNNING 5G system

On Worker3: start UPF

- cd NEW_5GC/free5gc

- ./bin/upf

On Worker: start webconsole

- cd free5gc/webconsole

- ./bin/webconsole

On Worker: start 5GC core

- cd free5gc

- ./run.sh

On Worker2 node: start GNB

- cd NEW_5GC/UERANSIM

- ./build/nr-gnb -c ./config/free5gc-gnb.yaml

On Worker2 node: start UE

- cd NEW_5GC/UERANSIM

- ./build/nr-ue -c ./config/free5gc-ue.yaml

- Step 2: RUNNING MININET NETWORK

On Worker3: delete routing rules if have

- route del -net 10.60.0.0/24 gw 192.168.0.20

On Mininet: restart network

- cd P4-MUP-SRv6

- make restart

- docker exec -it  mininet sh

- echo "deb http://archive.debian.org/debian stretch main" > /etc/apt/sources.list && chmod 1777 /tmp && rm -rf /var/lib/apt/lists/* && apt-get update && apt install bridge-utils

- make netcfg

- make app-reload

- bash run.sh

Then on Worker2

- ping 10.100.200.20

On Worker3:

- ping 192.168.0.20

On Worker1:

- ping 192.168.1.20

- Step3: Testing traffic from UE to UPF running in Worker3 and then terminate UPF.

On Worker2:

- ping 192.168.0.3 -I uesimtun0 -i 0.3

On Worker3:

- stop UPF running by Ctrl+C

- Step4: Running SRv6 MUP configuration

On Worker3: add routing rules

- route add -net 10.60.0.0/24 gw 192.168.0.20

On Mininet:

- bash srv6_mup.sh

- Step5: Access service 10.96.10.10 running at worker3 from UE

On Worker2:

- while sleep 1; do curl 10.96.10.10:5000 --interface uesimtun0; done

- Step6: Steer UE traffic from worker3 to worker1

On Mininet:

- bash util/onos-cmd route-gtp4d-insert device:r1 10.96.10.10 24 fcbb:bb00:01:: fcbb:bb00:6:7:9:fd44::

Notes:

- netplan config for internet connectivity:

- https://www.serverlab.ca/tutorials/linux/administration-linux/how-to-configure-networking-in-ubuntu-20-04-with-netplan/